Hydrogen Consumers: Metallurgical Chemical Industry and Resource Processing (MC19), and Pharmaceutical Manufacturing (PM19)

Gap 1: The "Two-Way Pipeline" Spatial Contradiction

In your current method, you find candidate hubs by drawing a flat 250 km radius buffer around points where three elements overlap: gas fields, hydrogen demand, and storage sinks.

- The Problem: Steam Methane Reforming (SMR) requires natural gas as an input feedstock, and produces hydrogen as an output product while generating CO2 as a waste stream. If your hub is placed exactly in the middle of a 250 km buffer, you may end up with a layout where you are building a 125 km pipeline to bring gas in, a 125 km pipeline to transport hydrogen out to consumers, and a 125 km pipeline to send CO2 back to the storage sink. Building three distinct, long-distance pipelines for input, output, and waste completely destroys the economic viability of the project.

- The Methodological Fix: In real engineering layouts, the SMR plant is always built directly on top of either the gas field or the storage sink to eliminate at least one massive pipeline leg entirely. Your optimization algorithm should be adjusted to test two specific, realistic configurations:

  - Gasfield-Gate SMR: Co-located at the gas wellhead (zero gas transport distance), piping CO2 to the sink and hydrogen to the market.

  - Sink-Gate SMR: Co-located at the injection wellhead (zero CO2 transport distance), piping raw natural gas in and hydrogen out.

To resolve Gap 1: The "Two-Way Pipeline" Spatial Contradiction, you must move away from standard distance overlays and introduce a Dual-Gate Co-Location Optimization Model.

Instead of an arbitrary multi-variable centroid, this methodology assumes that a Steam Methane Reforming (SMR) plant must be physically co-located either directly at the natural gas wellhead (Gasfield-Gate) or directly at the injection wellhead (Sink-Gate). This eliminates one entire long-distance pipeline leg, significantly dropping capital expenditure (CAPEX).