| SFPp=FAIS×exp-dλFAISmax×10 | (1) |
| --- | --- |

| SFP,raw,i=FAIS×exp-diλ×1+cosαi2 | (2) |
| --- | --- |

| SFP,i=SFP,raw,iFAISmax×10 | (3) |
| --- | --- |

Where αi is the acute angle (bounded between (0o and 90o) separating the shortest-path storage-to-fault vector from the structural strike azimuth of the nearest fault segment, FAISmax=10, representing the theoretical maximum hazard ceiling achievable within the FAIS.

The Mathematical/Spatial Logic (To extract the directional angle αi)

For each storage site point, the script will:

- Find the nearest line segment on the closest fault.

- Calculate the Strike Azimuth (vector angle) of that fault segment.

- Calculate the Azimuth of the shortest path vector connecting your storage site to that nearest fault point.

- Compute the acute difference angle \(\alpha _{i}\) (bounded between \(0^{\circ }\) and \(90^{\circ }\)) between those two vectors.

Here is the fully automated, deployment-ready Python script to run on your files:

import geopandas as gpd

import numpy as np

from shapely.ops import nearest_points

from shapely.geometry import LineString, Point

# 1. Load your shapefiles

# Replace paths with your actual filenames

storage_df = gpd.read_file("storage_potential_sites.shp")

faults_df = gpd.read_file("CAFD400_fault_lines.shp")

# 2. Project to an accurate coordinate system for China (CRITICAL for measuring angles/meters)

# We use China Geodetic Coordinate System 2000 / Albers Equal Area (EPSG:4479 or similar standard project)

projected_crs = "EPSG:4479"

storage_df = storage_df.to_crs(projected_crs)

faults_df = faults_df.to_crs(projected_crs)

def calculate_azimuth(x1, y1, x2, y2):

"""Calculates the absolute mathematical angle of a vector in radians."""

return np.arctan2(y2 - y1, x2 - x1)

def compute_alpha(site_geom, faults_gdf):

# Find the single nearest fault line geometry

nearest_fault = faults_gdf.geometry.iloc[faults_gdf.distance(site_geom).idxmin()]

# Get the closest point on that fault to our storage site

nearest_site_pt, nearest_fault_pt = nearest_points(site_geom, nearest_fault)

# --- Step A: Calculate Site-to-Fault Vector Azimuth ---

site_azimuth = calculate_azimuth(nearest_site_pt.x, nearest_site_pt.y,

nearest_fault_pt.x, nearest_fault_pt.y)

# --- Step B: Calculate Fault Line Segment Strike Azimuth ---

# Extract the coordinate vertices of the nearest fault line to find the local segment slope

coords = list(nearest_fault.coords)

# Find which segment vertex is closest to our touch point

distances = [Point(pt).distance(nearest_fault_pt) for pt in coords]

idx = np.argmin(distances)

# Get the two vertices forming the local fault segment

if idx == len(coords) - 1:

p1, p2 = coords[idx-1], coords[idx]

else:

p1, p2 = coords[idx], coords[idx+1]

fault_azimuth = calculate_azimuth(p1[0], p1[1], p2[0], p2[1])

# --- Step C: Compute the acute difference angle alpha ---

angle_diff = np.abs(site_azimuth - fault_azimuth)

# Convert to degrees

alpha_deg = np.degrees(angle_diff)

# Enforce standard acute geometry bounding (0 to 90 degrees)

alpha_deg = alpha_deg % 180

if alpha_deg > 90:

alpha_deg = 180 - alpha_deg

return alpha_deg

# 3. Iterate through every storage site and calculate alpha

print("Processing spatial vectors... This may take a moment.")

storage_df['alpha_deg'] = storage_df.geometry.apply(lambda geom: compute_alpha(geom, faults_df))

# 4. Save your updated storage data back to a new shapefile

# Convert back to your preferred original coordinate system if necessary (e.g., WGS84 - EPSG:4326)

storage_df = storage_df.to_crs("EPSG:4326")

storage_df.to_file("storage_potential_with_alpha.shp")

print("Successfully generated shapefile with 'alpha_deg' column!")

| Sensitivity Scenario | w1 | w2 | w3 |
| --- | --- | --- | --- |
| Baseline Case | 0.50 | 0.30 | 0.20 |
| Scenario A: Tectonic Dominance | 0.30 | 0.50 | 0.20 |
| Scenario B: Geotechnical Dominance | 0.40 | 0.20 | 0.40 |
| Scenario C: Absolute Shaking | 0.60 | 0.30 | 0.10 |